# Author Note 1

CEID algorithmic description, convergence, quantitative results, and supplementary pseudocode

# Replacement text

CEID was implemented as a constrained, closed-loop optimisation engine for autonomous colour formulation. A candidate formulation was represented by x = (vR, vY, vB, vW), where the variables are discrete transfer units of red, yellow, blue, and water. Each variable was restricted to an integer from 1 to 5, each unit represented 100 uL, and the total formulation was constrained by vR + vY + vB + vW <= 13, corresponding to a maximum mixture volume of 1,300 uL.

The experimentally captured target was prepared in well A1 using four red units, three yellow units, four blue units, and two water units. Its calibrated RGB response was R = 18, G = 20, and B = 11. Optimisation used the normalized eight-channel AS7341 spectrum rather than RGB alone.

| Wavelength (nm) | 415 | 445 | 480 | 515 | 555 | 590 | 630 | 680 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normalized target | 0.039722 | 0.319000 | 0.211944 | 0.242250 | 0.324583 | 0.391861 | 0.356694 | 0.182056 |

For each candidate x, the robotic platform prepared the formulation, collected and normalized the AS7341 spectrum, and represented it as Px = {(lambda_i, s_i(x))}. The scalar objective was f(x) = dF(Px, P*), the discrete Frechet distance between the measured and target spectral curves. The recurrence used was c(i,j) = max[d(pi,qj), min{c(i-1,j), c(i-1,j-1), c(i,j-1)}], with conventional boundary conditions. Smaller values indicate closer spectral agreement.

The search began with eight space-filling initial formulations. CEID then updated a probabilistic surrogate from completed experiments and selected 16 additional feasible formulations using a model-based acquisition policy. Experiments were executed in batches of four for a fixed budget of 24 physical experiments.

The production configuration did not implement numerical early stopping. Its operational stopping criterion was completion of the 24-experiment budget. The final incumbent was first discovered at trial 16. The best-so-far distance first fell below 0.03 at trial 14 and below 0.02 at trial 16; the remaining eight experiments did not improve the incumbent.

# Closed-loop result

CEID obtained its final best formulation at trial 16: five red units, one yellow unit, five blue units, and two water units, corresponding to 500, 100, 500, and 200 uL. A confirmatory remeasurement produced the corrected Frechet distance of 0.014524. The result export records the original trial-16 value as 0.022929 and identifies 0.014524 as the corrected remeasurement value. The formulation remained the incumbent through trial 24.

| Trial | Red | Yellow | Blue | Water | Frechet error | Best so far |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 1 | 5 | 2 | 0.032444 | 0.032444 |
| 2 | 1 | 4 | 1 | 4 | 0.036194 | 0.032444 |
| 3 | 2 | 2 | 4 | 1 | 0.118472 | 0.032444 |
| 4 | 3 | 1 | 2 | 4 | 0.039752 | 0.032444 |
| 5 | 2 | 3 | 5 | 1 | 0.082611 | 0.032444 |
| 6 | 1 | 1 | 2 | 5 | 0.133833 | 0.032444 |
| 7 | 5 | 2 | 2 | 1 | 0.098694 | 0.032444 |
| 8 | 1 | 4 | 4 | 4 | 0.087194 | 0.032444 |
| 9 | 4 | 1 | 4 | 3 | 0.035667 | 0.032444 |
| 10 | 1 | 5 | 1 | 4 | 0.134722 | 0.032444 |
| 11 | 1 | 4 | 1 | 2 | 0.141047 | 0.032444 |
| 12 | 2 | 4 | 1 | 5 | 0.091167 | 0.032444 |
| 13 | 3 | 1 | 4 | 3 | 0.063194 | 0.032444 |
| 14 | 4 | 1 | 3 | 2 | 0.024472 | 0.024472 |
| 15 | 4 | 1 | 2 | 4 | 0.042500 | 0.024472 |
| 16 | 5 | 1 | 5 | 2 | 0.014524* | 0.014524 |
| 17 | 5 | 1 | 3 | 2 | 0.069000 | 0.014524 |
| 18 | 5 | 1 | 4 | 3 | 0.036639 | 0.014524 |
| 19 | 5 | 1 | 4 | 2 | 0.092278 | 0.014524 |
| 20 | 5 | 3 | 4 | 1 | 0.067767 | 0.014524 |
| 21 | 3 | 1 | 3 | 2 | 0.075472 | 0.014524 |
| 22 | 4 | 1 | 5 | 3 | 0.048917 | 0.014524 |
| 23 | 3 | 1 | 3 | 3 | 0.055549 | 0.014524 |
| 24 | 4 | 1 | 2 | 2 | 0.153694 | 0.014524 |

* Corrected value from confirmatory remeasurement; original exported value = 0.022929.

Under the same reporting framework, the best physical distances were 0.017236 for deterministic grid search, 0.059319 for random search, 0.045000 for the strict-blind language-model-guided run, and 0.017375 for the resumed language-model-guided run. The resumed run contained 18 historical observations and six new physical experiments and must not be interpreted as an independent 24-experiment comparison.

# Supplementary pseudocode

Algorithm S1: CEID closed-loop spectral optimisation Input: target spectrum S*, feasible space X, N0 = 8, N = 24 1. Generate N0 feasible space-filling formulations X0. 2. Set experimental dataset D to empty. 3. For each x in X0: Prepare x; measure and normalize S(x). y <- discrete_frechet(S(x), S*). Append (x, y) to D. 4. While number_of_physical_experiments(D) < N: Fit or update the CEID probabilistic surrogate using D. Score feasible unevaluated formulations with the acquisition policy. Select the next constrained batch. For each selected x: Prepare x; measure and normalize S(x). y <- discrete_frechet(S(x), S*). Append (x, y) to D. 5. Return x_best = argmin(D.y), f_best = min(D.y), and D.

Reviewer binary: The reviewer-accessible WebGL binary and access statement are provided in Author Note 5.